Complex Urban Settlement Indices Implementation on ROI Image (Bahria Town Phase 7)

First we extract a landsat 8 image over Bahria Town Phase-7, Rawalpindi area, using a 2 km buffer and a time range from January 2023 to July 2025. The workflow begins by selecting the clearest Landsat 8 scene in the period, masking out clouds and invalid pixels, and exporting all 11 optical and thermal bands for analysis. It also generates a basic urban mask by combining NDBI and NDVI thresholds to identify built-up areas, which is exported as a separate raster. It incorporates NOAA’s VIIRS Nighttime Lights (NTL) monthly composites to compute both the maximum and 90th percentile brightness values across the time period, exporting both at 30-meter resolution to align with Landsat. For vegetation monitoring, the code calculates the Normalized Difference Vegetation Index (NDVI) for every Landsat scene, then computes and exports their pixelwise maximum values as separate layers. Additionally to calculate maximum LST (Land Surface Temperature) for Landsat 8, we also export annual maximum for these dates so that we could get annual maximum of all the bands. Finally, the script provides quick visual previews of the results and exports all processed images—including the Landsat stack, Landsat annual stack vegetation indice, urban mask, and NTL layers—to Google Drive for further analysis in Python or GIS. In short, this script automates the preparation of key urban and vegetation indicators for your study area, ready for advanced index computation and urban analysis. Dynamic World Labels classes were used as classes for the mask.  Dynamic World Labels has class names:

- Water

- Trees

- Grass

- Flooded Vegetation

- Crops

- Shrub & Scrub

- Built

- Bare

- Snow & Ice

The code used for extraction was:

/*********************  USER SETTINGS  **************************/

var BUFFER_METERS = 2000;                       // ~2 km radius

var DATE_START   = '2023-01-01';

var DATE_END     = '2025-07-01';

var CLOUD_THRESH = 10;                          // % cloud cover (scene-wide)

var DRIVE_FOLDER = 'GEE_Exports';               // change if you like

/****************************************************************/

// 1. Build filtered collection (as you already do)

var col = ee.ImageCollection('LANDSAT/LC08/C02/T1_TOA')

.filterBounds(roi)

.filterDate(DATE_START, DATE_END)

.filter(ee.Filter.lt('CLOUD_COVER', CLOUD_THRESH));

// 2. Mask clouds for each image in the collection

function maskClouds(img) {

var qa = img.select('QA_PIXEL');

var good = qa.bitwiseAnd(1 << 0).eq(0)

.and(qa.bitwiseAnd(1 << 1).eq(0))

.and(qa.bitwiseAnd(1 << 2).eq(0))

.and(qa.bitwiseAnd(1 << 3).eq(0));

return img.updateMask(good);

}

var col_masked = col.map(maskClouds);

// 3. Select the 11 bands

var bands = ['B1','B2','B3','B4','B5','B6','B7','B8','B9','B10','B11'];

// 4. Get pixelwise max for each band across all images in the collection

var maxBands = col_masked.select(bands).max().clip(roi);

// 5. Export the annual max composite

Export.image.toDrive({

image: maxBands,

description: 'L8_BahriaPhase7_AnnualMax11Bands',

folder: DRIVE_FOLDER,

fileNamePrefix: 'L8_BahriaPhase7_AnnualMax11Bands_' + DATE_START.replace(/-/g,'') + '_' + DATE_END.replace(/-/g,''),

region: roi,

scale: 30,

crs: 'EPSG:4326',

maxPixels: 1e10

});

// 6. QUICK URBAN MASK  (NDBI>0 & NDVI<0.2)

var ndvi = l8.normalizedDifference(['B5','B4']).rename('NDVI');

var ndbi = l8.normalizedDifference(['B6','B5']).rename('NDBI');

var urbanMask = ndbi.gt(0).and(ndvi.lt(0.2))

.rename('urban_mask');

// 7. VIIRS Nighttime Lights extraction (median composite for time range)

var viirs_col = ee.ImageCollection("NOAA/VIIRS/DNB/MONTHLY_V1/VCMCFG")

.filterBounds(roi)

.filterDate(DATE_START, DATE_END)

.select('avg_rad');

// 2. Create a per-pixel max composite (annual max)

var viirs_ntl_max = viirs_col.max().clip(roi);

// 3. Optionally, 90th percentile instead of strict max:

var viirs_ntl_p90 = viirs_col.reduce(ee.Reducer.percentile([90])).select('avg_rad_p90').clip(roi);

// 4. Export as before

Export.image.toDrive({

image: viirs_ntl_max,

description: 'VIIRS_NTL_MAX_BahriaPhase7',

folder: DRIVE_FOLDER,

fileNamePrefix: 'VIIRS_NTL_MAX_BahriaPhase7_' + DATE_START.replace(/-/g,'') + '_' + DATE_END.replace(/-/g,''),

region: roi,

scale: 30,

crs: 'EPSG:4326',

maxPixels: 1e10

});

Export.image.toDrive({

image: viirs_ntl_p90,

description: 'VIIRS_NTL_90p_BahriaPhase7',

folder: DRIVE_FOLDER,

fileNamePrefix: 'VIIRS_NTL_90p_BahriaPhase7_' + DATE_START.replace(/-/g,'') + '_' + DATE_END.replace(/-/g,''),

region: roi,

scale: 30,

crs: 'EPSG:4326',

maxPixels: 1e10

});

// 8. VISUAL PREVIEW

Map.centerObject(roi, 14);

Map.addLayer(allBands, {bands:['B4','B3','B2'], min:0.05, max:0.4}, 'L8 RGB');

Map.addLayer(urbanMask.updateMask(urbanMask), {palette:['black','red']}, 'Urban mask');

Map.addLayer(viirs_ntl_max, {min:0, max:50, palette:['black','blue','cyan','yellow','white']}, 'NTL (VIIRS)');

Map.addLayer(viirs_ntl_p90,{min:0, max:50, palette:['yellow']}, 'NTL (90 percent)');

// 9. EXPORT — 11-band image (Landsat)

Export.image.toDrive({

image: allBands,

description: 'L8_BahriaPhase7_11Bands',

folder: DRIVE_FOLDER,

fileNamePrefix: 'L8_BahriaPhase7_' + l8.date().format('yyyyMMdd').getInfo(),

region: roi,

scale: 30,

crs: 'EPSG:4326',

maxPixels: 1e10

});

// 10. EXPORT — binary urban mask (Landsat-based)

Export.image.toDrive({

image: urbanMask,

description: 'UrbanMask_BahriaPhase7',

folder: DRIVE_FOLDER,

fileNamePrefix: 'UrbanMask_BahriaPhase7_' + l8.date().format('yyyyMMdd').getInfo(),

region: roi,

scale: 30,

crs: 'EPSG:4326',

maxPixels: 1e10

});

// ******************************

//      USER SETTINGS

// ******************************

var BUFFER_METERS = 2000; // ~2 km radius

var DATE_START   = '2023-01-01';

var DATE_END     = '2025-07-01';

var CLOUD_THRESH = 10;    // % cloud cover

var DRIVE_FOLDER = 'GEE_Exports';

// ROI: Bahria Town Phase-7, Rawalpindi

var roi = ee.Geometry.Point(73.11988, 33.53081).buffer(BUFFER_METERS);

// ******************************

//      IMAGE COLLECTION

// ******************************

var col = ee.ImageCollection('LANDSAT/LC08/C02/T1_TOA')

.filterBounds(roi)

.filterDate(DATE_START, DATE_END)

.filter(ee.Filter.lt('CLOUD_COVER', CLOUD_THRESH));

// ******************************

//      NDVI CALCULATION

// ******************************

var addNDVI = function(img) {

var ndvi = img.normalizedDifference(['B5', 'B4']).rename('NDVI');

return img.addBands(ndvi);

};

var col_ndvi = col.map(addNDVI);

// ******************************

//      MAX COMPOSITES

// ******************************

var ndvi_max = col_ndvi.select('NDVI').max().clip(roi);

// ******************************

//      EXPORT TO DRIVE

// ******************************

Export.image.toDrive({

image: ndvi_max,

description: 'L8_NDVImax_BahriaPhase7',

folder: DRIVE_FOLDER,

fileNamePrefix: 'L8_NDVImax_BahriaPhase7_' + DATE_START.replace(/-/g,'') + '_' + DATE_END.replace(/-/g,''),

region: roi,

scale: 30,

crs: 'EPSG:4326',

maxPixels: 1e10

});

After downloading the images and masks. I viewed the images and masks:

import rasterio

import numpy as np

import matplotlib.pyplot as plt

from skimage.transform import resize

from matplotlib.colors import ListedColormap

import os

# ========== FILE PATHS ==========

landsat_path = r'C:\Users\Adeel Ajmal\Downloads\L8_BahriaPhase7_20231012.tif'

landsat_annual_path = r'C:\Users\Adeel Ajmal\Downloads\L8_BahriaPhase7_AnnualMax11Bands_20230101_20250701.tif'

label_path   = r'C:\Users\Adeel Ajmal\Downloads\DynamicWorld_LandcoverLabels.tif'

viirs_path   = r'C:\Users\Adeel Ajmal\Downloads\VIIRS_NTL_90p_BahriaPhase7_20230101_20250701.tif'

ndvi_max_path = r'C:\Users\Adeel Ajmal\Downloads\L8_NDVImax_BahriaPhase7_20230101_20250701.tif'

outdir = "urban_index_eval_complex_all"

os.makedirs(outdir, exist_ok=True)

# ========== 1. Landsat True Color ==========

with rasterio.open(landsat_path) as src:

rgb = src.read([4,3,2])  # Bands 4,3,2 = RGB

rgb = np.moveaxis(rgb, 0, -1)

def stretch(img):

img = img.astype('float32')

p2, p98 = np.nanpercentile(img, (2, 98))

return np.clip((img - p2) / (p98 - p2), 0, 1)

plt.figure(figsize=(7,6))

plt.imshow(stretch(rgb))

plt.title('Landsat-8 RGB (True Color)')

plt.axis('off')

plt.savefig(os.path.join(outdir, "landsat8_rgb.png"), bbox_inches='tight', dpi=200)

plt.show()

# ========== 2. Landsat Annual Max (visualize Band 4 as example) ==========

with rasterio.open(landsat_annual_path) as src:

band4 = src.read(11)  # Band 4 (Red) as an example

plt.figure(figsize=(7,6))

plt.imshow(band4, cmap='magma')

plt.title('Landsat-8 Annual Max (Band 11)')

plt.colorbar()

plt.axis('off')

plt.savefig(os.path.join(outdir, "landsat8_annualmax_band10.png"), bbox_inches='tight', dpi=200)

plt.show()

# ========== 3. Dynamic World Landcover Labels ==========

with rasterio.open(label_path) as src:

labels = src.read(1)

if labels.shape != rgb.shape[:2]:

labels_resized = resize(labels, rgb.shape[:2], order=0, preserve_range=True, anti_aliasing=False).astype(int)

else:

labels_resized = labels

class_palette = [

"#419bdf", "#397d49", "#88b053", "#7a87c6", "#e49635",

"#dfc35a", "#c4281b", "#a59b8f", "#ffffff"

]

plt.figure(figsize=(7,6))

plt.imshow(stretch(rgb))

plt.imshow(labels_resized, cmap=ListedColormap(class_palette), alpha=0.35, vmin=0, vmax=8)

plt.title('Landsat RGB + Dynamic World Labels')

plt.axis('off')

plt.savefig(os.path.join(outdir, "landsat_rgb_dw_labels.png"), bbox_inches='tight', dpi=200)

plt.show()

# ========== 4. VIIRS Nighttime Lights ==========

with rasterio.open(viirs_path) as src:

viirs = src.read(1)

plt.figure(figsize=(7,6))

plt.imshow(viirs, cmap='plasma')

plt.title('VIIRS Nighttime Lights (90th Percentile)')

plt.colorbar()

plt.axis('off')

plt.savefig(os.path.join(outdir, "viirs_ntl_90p.png"), bbox_inches='tight', dpi=200)

plt.show()

# ========== 5. NDVI Max ==========

with rasterio.open(ndvi_max_path) as src:

ndvi = src.read(1)

plt.figure(figsize=(7,6))

plt.imshow(ndvi, cmap='YlGn')

plt.title('NDVI Max (Landsat 8)')

plt.colorbar()

plt.axis('off')

plt.savefig(os.path.join(outdir, "ndvi_max.png"), bbox_inches='tight', dpi=200)

plt.show()

print("Done visualizing selected files!")

Output:

Landsat 8 Bands:

| Band Number | Band Name | Wavelength (µm) | Spatial Resolution (m) |
| --- | --- | --- | --- |
| 1 | Coastal /Aerosol | 0.43–0.45 | 30 |
| 2 | Blue | 0.45–0.51 | 30 |
| 3 | Green | 0.53–0.59 | 30 |
| 4 | Red | 0.64–0.67 | 30 |
| 5 | NIR | 0.85–0.88 | 30 |
| 6 | SWIR 1 | 1.57–1.65 | 30 |
| 7 | SWIR 2 | 2.11–2.29 | 30 |
| 8 | PAN (Panchromatic) | 0.50–0.68 | 15 |
| 9 | Cirrus | 1.36–1.38 | 30 |
| 10 | TIRS 1 (Thermal) | 10.6–11.19 | 100 |
| 11 | TIRS 2 (Thermal) | 11.50–12.51 | 100 |

Complex Urban Settlement Indices:

- Modified Normalized Difference Impervious Surface Index Liu (MNDISILiu)

- Vegetation Temperature Light Index (VTLI)

- Temperature and Vegetation Adjusted NTL Urban Index (TVANUI)

The formulas of these are:

Where:

- TLST : Land Surface Temperature (resampled to optical resolution),

- LLIT : Normalized VIIRS/ISS Nighttime Light Intensity,

- MNDWI: Modified Normalized Difference Water Index,

- ρSWIR1 : SWIR1 reflectance.

Where:

- NDVImax: annual maximum NDVI from Landsat 8 composites,

- Tmax: annual maximum monthly daytime Land Surface Temperature (LST),

- L: annual maximum VIIRS NTL

Where:

- LST: Normalized land surface temperature from Landsat 8,

- NDVI: Annual maximum NDVI from Landsat 8, normalized to [0, 1],

- NTL: VIIRS NTL data, normalized to [0, 1],

- ϵ: Small positive constant to prevent division by zero.

The code used for implementation is:

import rasterio

import numpy as np

import matplotlib.pyplot as plt

from matplotlib.colors import ListedColormap

from sklearn.metrics import (

jaccard_score, confusion_matrix, accuracy_score, precision_score,

recall_score, f1_score, balanced_accuracy_score, cohen_kappa_score,

roc_curve, auc

)

from skimage.transform import resize

import pandas as pd

import matplotlib.patches as mpatches

import os

# ======= Settings =======

landsat_path = r'C:\Users\Adeel Ajmal\Downloads\L8_BahriaPhase7_20231012.tif'

landsat_annual_path = r'C:\Users\Adeel Ajmal\Downloads\L8_BahriaPhase7_AnnualMax11Bands_20230101_20250701.tif'

label_path   = r'C:\Users\Adeel Ajmal\Downloads\DynamicWorld_LandcoverLabels.tif'

viirs_path   = r'C:\Users\Adeel Ajmal\Downloads\VIIRS_NTL_90p_BahriaPhase7_20230101_20250701.tif' # Update if needed

ndvi_max_path = r'C:\Users\Adeel Ajmal\Downloads\L8_NDVImax_BahriaPhase7_20230101_20250701.tif'

outdir       = "urban_index_eval_complex_all"

save_figs    = True

os.makedirs(outdir, exist_ok=True)

# ======= 1. Load Data =======

with rasterio.open(landsat_path) as src:

stack = src.read().astype('float32')

b1,b2,b3,b4,b5,b6,b7,b8,b9,b10,b11 = stack

with rasterio.open(landsat_annual_path) as src:

stack = src.read().astype('float32')

b1_annual,b2_annual,b3_annual,b4_annual,b5_annual,b6_annual,b7_annual,b8_annual,b9_annual,b10_annual,b11_annual = stack

with rasterio.open(label_path) as src:

labels = src.read(1)

with rasterio.open(viirs_path) as src:

viirs_ntl = src.read(1).astype('float32')

# Load annual max composites

with rasterio.open(ndvi_max_path) as src:

NDVI_max = src.read(1).astype('float32')

gt_mask = (labels == 6).astype(np.uint8)

gt_mask_resized = resize(

gt_mask,

(stack.shape[1], stack.shape[2]),

order=0, preserve_range=True, anti_aliasing=False

).astype(np.uint8)

# ======= 2. Define Indices (as per docx) =======

# Note: Add all requested indices here. All formulas are implemented below.

# For missing bands, formulas are adapted for Landsat 8. Each index is nan-masked if denominator can be zero.

def safe_div(num, denom):

out = np.full_like(num, np.nan)

valid = np.abs(denom) > 1e-6

out[valid] = num[valid] / denom[valid]

return out

indices = {}

# ---------- EXTRA PREP: Reflectance ratio bands (already float32) ----------

# Assumption: stack contains TOA reflectance for optical, TOA BT (Kelvin) for thermal.

rho_red  = b4          # reflectance red

rho_nir  = b5

rho_swir1= b6

rho_green= b3

rho_swir2= b7

rho_blue = b2

# ---------- 1. NDVI, MNDWI (unchanged) ----------

NDVI = safe_div(rho_nir - rho_red, rho_nir + rho_red)

MNDWI = safe_div(rho_green - rho_swir1, rho_green + rho_swir1)

NDWI = safe_div(rho_green - rho_nir, rho_green + rho_nir)

indices['NDVI'] = NDVI

indices['MNDWI'] = MNDWI

# ======= 4. Normalize VIIRS NTL =======

ntl_mask = np.isfinite(viirs_ntl) & (viirs_ntl > 0)

NTL_min = np.nanmin(viirs_ntl[ntl_mask])

NTL_max = np.nanmax(viirs_ntl[ntl_mask])

NTL_norm = np.zeros_like(viirs_ntl)

NTL_norm[ntl_mask] = (viirs_ntl[ntl_mask] - NTL_min) / (NTL_max - NTL_min)

NTL_norm[~ntl_mask] = 0

# Masked NTL for urban (optional, not used in index formulas, but you can analyze separately)

NTL_urban = NTL_norm * gt_mask_resized

# ======= Implement Urban NTL Indices (add to indices dictionary) =======

# ---- Emissivity (Sun et al. 2019) ----

Pv = ((NDVI - 0.2) / 0.3)**2

ε = np.where(

NDVI < 0.2,

0.979 - 0.035 * rho_red,

np.where(

NDVI <= 0.5,

0.986 + 0.004 * Pv,

0.990

)

)

ε = np.clip(ε, 0.85, 0.999)  # keep within physical bounds

# ---- LST (Kelvin, Sun et al. Eq. 56) ----

λ = 10.8e-6        # central wavelength (m) for L8 band 10

ρ_const = 1.438e-2 # m·K

Tb = b10           # Band 10: TOA Brightness Temp in Kelvin

LST = Tb / (1 + (λ * Tb / ρ_const) * np.log(ε))

LST_norm = (LST - np.nanmin(LST)) / (np.nanmax(LST) - np.nanmin(LST) + 1e-6)

# --- MNDSI_Liu ---

rho_swir1_norm = (rho_swir1 - np.nanmin(rho_swir1)) / (np.nanmax(rho_swir1) - np.nanmin(rho_swir1) + 1e-6)

num = LST + NTL_norm - (MNDWI + rho_swir1_norm)

den = LST + NTL_norm + MNDWI + rho_swir1_norm + 1e-6

MNDSI_Liu = num / den

indices['MNDSI_Liu'] = MNDSI_Liu

# --- VTLI ---

Pv = ((NDVI_max - 0.2) / 0.3)**2

ε = np.where(

NDVI_max < 0.2,

0.979 - 0.035 * b4_annual,

np.where(

NDVI_max <= 0.5,

0.986 + 0.004 * Pv,

0.990

)

)

ε = np.clip(ε, 0.85, 0.999)  # keep within physical bounds

# ---- LST (Kelvin, Sun et al. Eq. 56) ----

λ = 10.8e-6        # central wavelength (m) for L8 band 10

ρ_const = 1.438e-2 # m·K

Tb = b10_annual           # Band 10: TOA Brightness Temp in Kelvin

LST_max = Tb / (1 + (λ * Tb / ρ_const) * np.log(ε))

VTLI = (1 - NDVI_max) * LST_max * NTL_norm

indices['VTLI'] = VTLI

# --- TVANUI ---

epsilon = 1e-6

LST_div = LST_norm / (NDVI_max + epsilon)

arctan_part = np.arctan(LST_div) / (np.pi / 2)

NTL_063 = NTL_norm

TVANUI = arctan_part * NTL_063

indices['TVANUI'] = TVANUI

# --- LST itself (optional visualization) ---

indices['LST'] = LST

indices['LST_max'] = LST_max

# ======= 7. NDVI Threshold Search & Best Mask =======

ndvi_thresholds = np.arange(0.1, 0.81, 0.05)

best_results = {}

all_results = {}

def best_threshold(idx, truth, n_steps=200):

v = np.isfinite(idx) & np.isfinite(truth)

x, y = idx[v].ravel(), truth[v].ravel()

best_thr, best_iou = None, -1

for thr in np.linspace(np.nanmin(x), np.nanmax(x), n_steps):

y_pred = (x > thr).astype(np.uint8)

if y_pred.sum() == 0: continue

iou = jaccard_score(y, y_pred, zero_division=0)

if iou > best_iou: best_thr, best_iou = thr, iou

return best_thr

def metrics(mask_pred, truth, idx_pred_scores=None):

v = np.isfinite(mask_pred) & np.isfinite(truth)

y_pred, y_true = mask_pred[v].ravel(), truth[v].ravel()

out = dict(

IoU=jaccard_score(y_true, y_pred, zero_division=0),

F1=f1_score(y_true, y_pred, zero_division=0),

Pr=precision_score(y_true, y_pred, zero_division=0),

Re=recall_score(y_true, y_pred, zero_division=0),

Acc=accuracy_score(y_true, y_pred),

BAc=balanced_accuracy_score(y_true, y_pred),

Kappa=cohen_kappa_score(y_true, y_pred)

)

tn, fp, fn, tp = confusion_matrix(y_true, y_pred, labels=[0,1]).ravel()

out.update({"TP":tp,"TN":tn,"FP":fp,"FN":fn})

if idx_pred_scores is not None:

v2 = v & np.isfinite(idx_pred_scores)

y_true2 = truth[v2].ravel()

y_score2 = idx_pred_scores[v2].ravel()

if len(np.unique(y_true2)) > 1 and len(y_score2) > 0:

try:

fpr, tpr, _ = roc_curve(y_true2, y_score2)

out["AUC"] = auc(fpr, tpr)

except Exception as e:

out["AUC"] = np.nan

else:

out["AUC"] = np.nan

else:

out["AUC"] = np.nan

return out

for idx_name, idx in indices.items():

metric_by_thr = []

for ndvi_thr in ndvi_thresholds:

thr = best_threshold(idx, gt_mask_resized)

pred = ((idx > thr) & (NDVI < ndvi_thr)).astype(np.uint8)

result = metrics(pred, gt_mask_resized, idx_pred_scores=idx)

result['ndvi_thr'] = ndvi_thr

result['idx_thr'] = thr

metric_by_thr.append(result)

df_thr = pd.DataFrame(metric_by_thr)

best_i = df_thr['IoU'].idxmax()

best_results[idx_name] = df_thr.loc[best_i].to_dict()

all_results[idx_name] = df_thr

# ======= 8. Visualization: 3-panel for Each Index =======

class_palette = [

"#419bdf", "#397d49", "#88b053", "#7a87c6", "#e49635",

"#dfc35a", "#c4281b", "#a59b8f", "#ffffff"

]

class_names = [

"Water", "Trees", "Grass", "Flooded Veg", "Crops",

"Shrub/Scrub", "Built", "Bare", "Snow/Ice"

]

label_viz = resize(labels, (stack.shape[1], stack.shape[2]), order=0, preserve_range=True, anti_aliasing=False)

rgb = np.stack([b4, b3, b2], -1)

p2, p98 = np.nanpercentile(rgb, (2, 98))

rgb = np.clip((rgb - p2) / (p98 - p2), 0, 1)

rgb[np.isnan(rgb)] = 0

URBAN_COLOR = np.array([1.0, 0.1, 0.1])

NON_URBAN_COLOR = np.array([0.65, 0.65, 0.65])

for idx_name, idx in indices.items():

best = best_results[idx_name]

ndvi_thr = best['ndvi_thr']

idx_thr = best['idx_thr']

pred = ((idx > idx_thr) & (NDVI < ndvi_thr)).astype(np.uint8)

fig, axs = plt.subplots(1, 3, figsize=(22, 7))

axs[0].imshow(rgb)

axs[0].set_title("Landsat-8 RGB")

axs[0].axis('off')

im2 = axs[1].imshow(label_viz, cmap=ListedColormap(class_palette), vmin=0, vmax=8)

axs[1].set_title("Dynamic World Labels")

axs[1].axis('off')

cbar = plt.colorbar(im2, ax=axs[1], ticks=np.arange(0,9), shrink=0.8)

cbar.ax.set_yticklabels(class_names)

idx_disp = np.copy(idx)

idx_disp[~np.isfinite(idx_disp)] = np.nan

axs[2].imshow(idx_disp, cmap='RdYlBu')

gt_viz = np.zeros((*gt_mask_resized.shape, 3), dtype=np.float32)

gt_viz[gt_mask_resized == 1] = URBAN_COLOR

gt_viz[gt_mask_resized == 0] = NON_URBAN_COLOR

pred_viz = np.zeros((*pred.shape, 3), dtype=np.float32)

pred_viz[pred == 1] = URBAN_COLOR

pred_viz[pred == 0] = NON_URBAN_COLOR

axs[2].imshow(0.6*gt_viz + 0.4*pred_viz, alpha=0.45)

axs[2].set_title(f"{idx_name} Index\nIoU={best['IoU']:.3f} NDVI<{ndvi_thr:.2f}")

axs[2].axis('off')

handles = [

mpatches.Patch(color=URBAN_COLOR, label='Urban'),

mpatches.Patch(color=NON_URBAN_COLOR, label='Non-Urban')

]

axs[2].legend(handles=handles, loc='lower right', fontsize=12)

plt.tight_layout()

if save_figs:

plt.savefig(os.path.join(outdir, f"{idx_name}_urban3panel.png"), dpi=200, bbox_inches='tight')

plt.show()

# ======= 9. Save Summary Table =======

header = "{:<10} {:>8} {:>8} {:>8} {:>8} {:>8} {:>8} {:>8} {:>8} {:>6} {:>8}".format(

"Index","IoU","F1","Pr","Re","Acc","BAc","Kappa","AUC","TP","NDVI_thr")

print(header)

print("-"*len(header))

for k,v in best_results.items():

print("{:<10} {:8.3f} {:8.3f} {:8.3f} {:8.3f} {:8.3f} {:8.3f} {:8.3f} {:8.3f} {:6.0f} {:8.2f}".format(

k, v['IoU'], v['F1'], v['Pr'], v['Re'], v['Acc'], v['BAc'], v['Kappa'], v['AUC'], v['TP'], v['ndvi_thr']))

df = pd.DataFrame(best_results).T

df.to_csv(os.path.join(outdir, "urban_index_metrics_best.csv"))

for k, df_thr in all_results.items():

df_thr.to_csv(os.path.join(outdir, f"{k}_ndvi_threshold_sweep.csv"))

print("GT Urban Fraction: {:.2%}".format(gt_mask_resized.mean()))

vals, counts = np.unique(label_viz.astype(int), return_counts=True)

total = label_viz.size

print("\n--- Land Cover Breakdown (Dynamic World) ---")

for v, c in zip(vals, counts):

print(f"{class_names[v]:<12}: {c/total*100:5.2f}%")

import numpy as np

import matplotlib.pyplot as plt

from sklearn.metrics import confusion_matrix

import seaborn as sns

import os

def plot_confusion_matrix(y_true, y_pred, idx_name, outdir, labels=['Non-Urban', 'Urban']):

cm = confusion_matrix(y_true, y_pred, labels=[0,1])

plt.figure(figsize=(4,4))

sns.heatmap(cm, annot=True, fmt='d', cmap='Blues',

xticklabels=labels, yticklabels=labels,

cbar=False)

plt.xlabel('Predicted Label')

plt.ylabel('True Label')

plt.title(f'{idx_name} - Confusion Matrix')

plt.tight_layout()

# Save the figure

plt.savefig(os.path.join(outdir, f"{idx_name}_confusion_matrix.png"), dpi=150)

plt.close()

# Main loop: Go through all indices

for idx_name, idx in indices.items():

best = best_results[idx_name]

ndvi_thr = best['ndvi_thr']

idx_thr = best['idx_thr']

pred = ((idx > idx_thr) & (NDVI < ndvi_thr)).astype(np.uint8)

y_true = gt_mask_resized.ravel()

y_pred = pred.ravel()

plot_confusion_matrix(y_true, y_pred, idx_name, outdir)

print(f"Confusion matrix PNGs saved to {outdir}")

Output:

MNDSILIU Result:

VTLI Result:

TVANUI Result:

Metrics Result For Each Index:

| Index Name | IoU-score | F1-score | Precision | Recall | Accuracy | Balanced Accuracy Score | Kappa Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MNDSILIU | 0.891773 | 0.942791 | 0.892885 | 0.998606 | 0.914832 | 0.85769 | 0.778233 |
| VTLI | 0.893337 | 0.943664 | 0.899627 | 0.992235 | 0.916744 | 0.865253 | 0.785672 |
| TVANUI | 0.892476 | 0.943184 | 0.897018 | 0.994359 | 0.915812 | 0.862235 | 0.78238 |

| Index Name | True Positives | True Negatives | False Positives | False Negatives | Area Under the Curve | NDVI threshold |
| --- | --- | --- | --- | --- | --- | --- |
| MNDSILIU | 15046 | 4568 | 1805 | 21 | 0.34584 | 0.75 |
| VTLI | 14950 | 4705 | 1668 | 117 | 0.729164 | 0.75 |
| TVANUI | 14982 | 4653 | 1720 | 85 | 0.781851 | 0.75 |

Confusion Matrices of Indices: